# Equipment Standard Operating Procedures

Site / activity: Squirrel Wood Scout Campsite — Chainsaws, Iseki tractor, mowers, Polaris ATV, strimmers and log splitter

Prepared by: Andy McAteer, Support Team Lead

Date: 27 March 2026

Review: Annually, or after incident, defect, training update or equipment change

| Hazard / activity | People involved | Control measures / procedure | Review / notes |
| --- | --- | --- | --- |
| Chainsaws | Trained and authorised adults only. | Purpose: Cutting and processing timber by competent authorised users only. Location: Squirrel Wood Scout Campsite Required PPE: Helmet with visor and hearing protection, chainsaw trousers, gloves, chainsaw boots. |  |
| Chainsaws Pre-use checks | Trained and authorised adults only. | • Competence confirmed and task briefed • Saw, chain, brake and bar checked • Fuel and chain oil available • Work area, timber tension and escape routes checked • Exclusion zone set and first aid available |  |
| Chainsaws Safe operation | Trained and authorised adults only. | • Use a recognised safe start method, never drop start • Keep both hands on the saw and maintain secure footing • Use correct cutting technique and watch for kickback • Stop if anyone enters the exclusion zone • Take breaks and stop if tired or conditions change |  |
| Chainsaws Post-use and storage | Trained and authorised adults only. | • Switch off before moving, adjusting or clearing • Allow to cool before refuelling or transport • Clean, inspect and report defects • Store securely with bar cover fitted |  |
| Chainsaws Emergency action | Trained and authorised adults only. | If injury, fire, chain failure or unstable timber occurs, stop work, make the area safe and follow the site emergency plan. | Stop work immediately if the equipment becomes unsafe, weather or terrain changes, or an unauthorised person enters the work area. |
| Iseki Tractor | Trained and authorised adults only. | Purpose: Safe use of the Iseki tractor for towing, transport and grounds tasks. Location: Squirrel Wood Scout Campsite Required PPE: Sturdy footwear, suitable workwear, hearing protection if needed. |  |
| Iseki Tractor Pre-use checks | Trained and authorised adults only. | • Walk-round check for tyres, steering, brakes, leaks and guards • ROPS and seatbelt serviceable • Fuel, oil and coolant checked • Trailer or attachment correctly fitted and within limits • Route, slope and pedestrian risks assessed |  |
| Iseki Tractor Safe operation | Trained and authorised adults only. | • Mount and dismount using handholds • Drive slowly and to site conditions • Avoid unsafe slopes, sharp turns and harsh braking • Use a banksman where visibility is restricted • Keep pedestrians clear and do not carry unauthorised passengers |  |
| Iseki Tractor Post-use and storage | Trained and authorised adults only. | • Park safely, lower implements and apply brake • Switch off, remove key and secure it • Report damage, defects or near misses • Leave tractor and attachments tidy and safe |  |
| Iseki Tractor Emergency action | Trained and authorised adults only. | If rollover, collision or loss of control occurs, stop, isolate if safe and summon assistance immediately. | Stop work immediately if the equipment becomes unsafe, weather or terrain changes, or an unauthorised person enters the work area. |
| Mowers | Trained and authorised adults only. | Purpose: Safe use of pedestrian and ride-on mowers for grounds maintenance. Location: Squirrel Wood Scout Campsite Required PPE: Sturdy footwear, eye protection where needed, hearing protection if required. |  |
| Mowers Pre-use checks | Trained and authorised adults only. | • Guards, blades, controls and fuel or battery condition checked • Area cleared of stones, branches, litter and hidden obstacles • Slopes and soft ground assessed • Bystanders kept clear before starting |  |
| Mowers Safe operation | Trained and authorised adults only. | • Use the mower in line with manufacturer guidance • Keep hands and feet away from blades and discharge areas • Work at a controlled pace and take care on slopes and edges • Stop blades and isolate before clearing blockages • Cease work if people enter the danger area |  |
| Mowers Post-use and storage | Trained and authorised adults only. | • Switch off and wait for moving parts to stop • Clean down safely using suitable tools • Refuel only when cool and in a safe place • Report defects and store securely |  |
| Mowers Emergency action | Trained and authorised adults only. | For injury, fire, debris strike or loss of control, stop the mower, isolate it and follow site emergency arrangements. | Stop work immediately if the equipment becomes unsafe, weather or terrain changes, or an unauthorised person enters the work area. |
| Polaris ATV | Trained and authorised adults only. | Purpose: Controlled use of the Polaris ATV for transport and light utility work. Location: Squirrel Wood Scout Campsite Required PPE: Suitable workwear, sturdy footwear and gloves if required. |  |
| Polaris ATV Pre-use checks | Trained and authorised adults only. | • Vehicle checked for tyres, brakes, lights, steering and leaks • Loads, trailer and hitching arrangements suitable • Route, terrain, weather and pedestrian risks assessed • Keys controlled and available only to authorised users |  |
| Polaris ATV Safe operation | Trained and authorised adults only. | • Drive at safe speeds for the ground and task • Use seatbelts where fitted • Do not carry passengers unless the vehicle is designed for them • Secure loads and do not overload • Use a banksman when manoeuvring in tight or busy areas |  |
| Polaris ATV Post-use and storage | Trained and authorised adults only. | • Park safely, apply brake and switch off • Remove and secure the key • Complete any defect or incident reporting • Leave the vehicle clean and ready for authorised use |  |
| Polaris ATV Emergency action | Trained and authorised adults only. | If the ATV overturns, collides or develops a fault, stop use immediately, make the area safe and report it at once. | Stop work immediately if the equipment becomes unsafe, weather or terrain changes, or an unauthorised person enters the work area. |
| Strimmers | Trained and authorised adults only. | Purpose: Safe strimming and brush cutting while controlling debris, cuts and exposure risks. Location: Squirrel Wood Scout Campsite Required PPE: Face or eye protection, hearing protection, gloves, boots and long trousers. |  |
| Strimmers Pre-use checks | Trained and authorised adults only. | • Guards, cutting head, controls and harness checked • Area inspected for wire, stones, glass and hidden obstacles • Machine suitable for the vegetation being cut • Strict exclusion zone established |  |
| Strimmers Safe operation | Trained and authorised adults only. | • Use the correct starting method and maintain two-handed control • Keep the cutting head away from feet and legs • Direct debris away from people, vehicles and buildings • Stop work immediately if anyone enters the exclusion zone • Take breaks to manage fatigue and vibration exposure |  |
| Strimmers Post-use and storage | Trained and authorised adults only. | • Switch off and wait for the head to stop • Clean build-up safely and inspect for wear or damage • Refuel only when cool and away from ignition sources • Store securely and report defects |  |
| Strimmers Emergency action | Trained and authorised adults only. | If there is fuel spillage, fire, loss of control or injury, stop work, isolate the machine and follow emergency procedures. | Stop work immediately if the equipment becomes unsafe, weather or terrain changes, or an unauthorised person enters the work area. |
| Log Splitter | Trained and authorised adults only. | Purpose: Safe splitting of timber using the approved site log splitter. Location: Squirrel Wood Scout Campsite Required PPE: Eye protection, gloves for timber handling and sturdy footwear. |  |
| Log Splitter Pre-use checks | Trained and authorised adults only. | • Machine on stable ground with a clear work area • Controls, hoses, guards and frame checked • Two-hand controls operating correctly • Timber suitable in size and free from obvious metal or foreign objects |  |
| Log Splitter Safe operation | Trained and authorised adults only. | • Keep hands clear of the splitting zone at all times • Place logs squarely before activating the splitter • Do not hold unstable timber close to the ram or wedge • Stop the machine before clearing jams or making adjustments • Keep the area tidy to prevent slips and poor lifting |  |
| Log Splitter Post-use and storage | Trained and authorised adults only. | • Isolate and leave controls in a safe state • Check for hydraulic leaks or damage • Clear timber and offcuts from the work area • Store securely and report defects |  |
| Log Splitter Emergency action | Trained and authorised adults only. | If a crush injury, jam, unstable log movement or hydraulic failure occurs, stop immediately and seek help. | Stop work immediately if the equipment becomes unsafe, weather or terrain changes, or an unauthorised person enters the work area. |

These SOPs are minimum site rules and must be read alongside the relevant risk assessment, manufacturer guidance and any task-specific briefing.

Reviewed by: ____________________    Date: ____________________    Changes: ____________________

Checked by Operations Manager: ____________________    Date: ____________________